# Information: Please use the file CCN_Lab1_Template.gns3project
Rename the file if necessary by removing the suffix .seure

# Lab 1

Enter your data here:

| Group DAY_XX | Name | Forename | Matr. No. |
| --- | --- | --- | --- |
| Member #1 |  |  |  |
| Member #2 |  |  |  |
| Member #3 |  |  |  |

Introduction: As part of the lab, you will develop a network for a start-up company.  You are the CTO of the start-up company and have big goals, so the network will grow rapidly over time. Accordingly, decisions must be made and tested about the network design as well as the individual configurations of the network elements. Thus, you will expand this network step by step with further requirements and functionalities to achieve your goals.

Initially, your start-up company's network consists of three hosts and a hub. The hosts are based on Debian (a Linux system) and have some networking tools that will be used during the lab.

In this first lab, you will get to know and understand the given network by looking at the individual configurations using various network tools.

## Task 1:

Determine the HW address and the IPv4 address for each host, as well as the name of the interface, if available.

To open the console, right-click on each host, and then select “Console” after you started the host. After the TightVNC Viewer has opened, the console can be reached via the "JWM" area (bottom left, select Terminal).

Command:

| Interface | MAC (HW address) | IPv4 address |
| --- | --- | --- |

What is the difference between a MAC address and an IP address?

## Task 2:

If necessary, add static IPv4 addresses to the hosts that do not have an IPv4 address.

To do this, edit the file /etc/network/interfaces with the editor nano by entering an IPv4 address and subnet mask in the file. Select a suitable IPv4 address similar to the remaining hosts, compare the configurations of the other hosts as well. As a netmask use 255.255.255.0 and the broadcast address is 192.168.11.255.

Tips:

- To open: nano <path including filename>

- The # character comments out the line in the file.

- The file can be saved using Ctrl+O (do not change file name!) and Ctrl+X exits the editor.

What would be the command in the command line (not in the editor nano) to assign an IP address and the netmask?

Command:

## Task 3:

Now test the accessibility between the individual hosts.

Commands:

Result:

## Task 4:

Which command can be used to see all the temporarily cached HW addresses of other hosts, on one host? (ARP cache)

Command:

Which IP addresses/HW addresses can be seen on the first host?

| IPv4 address | MAC (HW address) | Interface |
| --- | --- | --- |

Now delete the entry from the third host in the ARP cache of the first host.

Command:

Now ping again from the first host to the third host. Log this out using Wireshark by right-clicking the connection between the host and hub, and then selecting “Start capture”. The following window can be confirmed with OK, so that Wireshark starts automatically, and the recording begins.

Command:

Which protocols can be seen in Wireshark under the tab "Protocol"?  Insert a small screenshot of Wireshark or alternatively a list here.

Is the ARP response frame a broadcast frame?

What are the tasks of the protocols you see there?

## Task 5:

Now ping the first and third hosts again. This time, log all three connections on the hub with the help of Wireshark.

Command:

What do you notice about the three connections or what problem could arise?

How could the problem be solved or with which network device could the problem be solved?

## Task 6:

Next, replace the hub by a switch by right clicking the hub and selecting Delete. Then add a switch from the left menu under “Switches” and connect all hosts (e.B. Host1 <-> Ethernet1, etc.).

Now ping the first and third hosts again. This time, log all three connections on the switch with the help of Wireshark.

Command:

What do you notice about the three connections?

## Task 7:

With the switch, you now have the possibility to divide the employees into virtual networks according to their departments.

Now add the first two hosts to VLAN 11 and the third host to VLAN 12.

To configure the switch, right-click the switch and select Configure. There you can assign VLANs to the individual ports.

Now test the accessibility between the individual hosts.

Commands:

Result: