# Lab 3

Enter your data here:

| Group DAY_XX | Name | Forename | Matr. No. |
| --- | --- | --- | --- |
| Member #1 |  |  |  |
| Member #2 |  |  |  |
| Member #3 |  |  |  |

Your start-up company continues to grow and the network of lab 2 is expanded. In this lab, we deal with the network that lies outside the two locations.

Important: To start with Lab 3 the tasks you need a fully working solution of Lab 2! 
If you use subinterfaces for the interconnection of the VLANs in LAB2, change this configuration and use virtual interfaces for the VLANs 11 and 12 on R1 and for VLAN 21 and 22 on R2. Otherwise, you won’t be able to finish this lab successfully.

The tasks assume the following network configuration.

VLANs 21 and 22 Fa0/0

VLANs 11 and 12 Fa0/0

10.0.0.2 Fa0/1

Fa0/1 10.0.0.1

Location 1

(Network from P1)

Location 2

Router R1

Router R2

Fig.: Sketch of the network before task 1

## Task 1:

Add an additional router R3 that connects both routers from the last lab.  Add a new network module card to each router. To do this, do a right-click on the router icon and choose “Configure”. Choose the tab slots and add the NM-4E card to slot 1.

Now connect the R3 with R1 over ethernet 1/0 and R3 with R2 over ethernet1/1

E1/1 10.0.2.1

E1/1 10.0.2.2

10.0.1.2 E1/0

E1/0 10.0.1.1

10.0.0.2 Fa 0/1

Fa0/1 10.0.0.1

Router R2

Router R1

Router R3

To do this, configure the corresponding interfaces of the three routers in such a way that a triangular connection is created. Use a network 10.0.X.0/30 analogous to lab 2 (see figure below).

## Task 2:

Configure the network between R1 and R3 with the subnet 10.0.1.0/30 and the network between R2 and R3 with the subnet 10.0.2.0 /30.

Command:

## Task 3:

Now set up a DHCP server on R1, which assigns only the upper half of the IP addresses in each network (VLAN11, VLAN12, VLAN21, VLAN22). Also define the netmask, router address and DNS server. The DNS server is the second router, use the ip address in the VLAN 21 for this.

Important: The assignment of the IP address may take some time.

Commands:

DHCP config:

| Virtual Interface | VLAN | IP Address Router | DHCP Range | Netmask |
| --- | --- | --- | --- | --- |

Which command can be used to output the gateway of a host, if it is assigned by a DHCP server?

Command:

Set up all hosts that they can now be configured via DHCP. To do this, edit the file /etc/network/interfaces again with the editor nano.

Now turn off the first host and start Wireshark to record the connection from the first host. In the search, set the capture filter to udp to display all DHCP packets.

Then start the first host and analyze the output in Wireshark.

Enter the received IP configuration of eth0 here:

| Host | IPv4 address | Netmask | Gateway | DNS server |
| --- | --- | --- | --- | --- |

Which DHCP packets are exchanged in which order?

## Task 4:

Configure R2 as the name server for the entire network.

Command:

Configure the following mapping:

IP-address of H1 as with the name host1

IP-address of H2 as with the name host2

Command:

As a result, all clients should be able to use the name instead of the IP-address.

Command:

Result:

What is a possible problem if you add such a configuration?

Result:

Add a third entry with the mapping name = web, IP-address = 172.16.0.100

Command:

Perform an echo request from H1 to the name web. What is the output?

Result:

## Task 5:

Are all systems able to receive an IP-address from R1?

Result:

Locate the problem inside the network. Which connections should be checked for this?

Result:

What is necessary to facilitate a working DHCP in the network? Configure the network properly.

Command:

After the correct configuration, turn off the first host in VLAN 21 and start Wireshark to record the connection from this host. In the search, set the capture filter to dhcp to display all DHCP packets.

Then start this host and analyze the output in Wireshark.

What differentiates from the capture of task 3?

Result:

Enter the received IP configuration of eth0 of H5 and H6 here:

| Host | IPv4 address | Netmask | Gateway | DNS server |
| --- | --- | --- | --- | --- |

## Task 6:

Connect a web server from the menu on the left to the third router by using a switch between the router and the web server. However, you do not need to configure anything else on this switch. For the interface of R3 please use 172.16.0.1/12. For the IP address for the web server please use 172.16.0.100/12.

Show the relevant part of the /etc/network/interfaces file here:

## Task 7:

Is H1 able to ping the webserver?

Result:

Perform a traceroute to the IP-address and copy the output in here:

Command:

Result:

## Task 8:

Now add the dynamic routing protocol RIP to the configuration of the three routers and configure the correct setup for an exchange of the network information between the routers.

Command:

Check the resulting routing tables of R1, R2 and R3 and copy the output in here:

Result:

Explain the entries of R3:

Result:

Now check the accessibility of the web server from H1, H3, H4 and H6.

Command:

Result:

Perform a traceroute from H1 to the webserver and copy the output in here:

Command:

Result:

## Task 9:

Now start Wireshark on the connection between R3 and R1 and then check the connection using Firefox on Host 3 by calling http://172.16.0.100 and http://web. Copy a screenshot of the relevant packets:

Result:

## Task 10:

Limit the access to the webserver, so that H1 can ping the webserver, but cannot request the http process on port 80.

Command:

Now test the rules by pinging from H1, H3 and H4. What is allowed and what does not work?

Tests:

After that, test the rules by requesting the website from H1, H3 and H4. What is allowed and what does not work?

Tests:

On the router, look at the statistics of the access lists and interpret them.

Commands:

# Appendix

Commands for the host network functions. Commands without parameters and options (see Linux an-pages):

| Command | Description |
| --- | --- |
| ifconfig | Configuration and control of IP network interface |
| ping | Accessibility in the IP network |
| arp | Display and modification of ARP1 cache |
| route | Viewing / Changing the IP Routing Table |

1ARP = Address Resolution Protocol: Mapping of IP addresses to HW addresses in the local network

Commands for the Cisco Router:

| Befehl | Description |
| --- | --- |
| ? | shows a list of possible commands/alternatives or parameters depending on the context. |
| exit | ends the session or goes one mode back |
| end | decreases one command level |
| no ... | negates (deletes) the following command line |
| show [Parameter] | displays information |
| show version | Displays information about the router |
| show running-config | displays the ongoing configuration of the switch and router |
| show interfaces | displays all information about the interfaces |
| show interfaces status | displays a quick overview (status) of the interfaces |
| configure terminal | switches to configuration mode |
| int fastethernet x/x | select the interface to be configured in configuration mode |
| switchport mode trunk | Sets the port to trunk mode |
| interface VLAN n | Definition/selection of the virtual router interface to be configured in configuration mode (n=VLAN No.) |
| vlan database | Managing VLANs of a router |
| vlan x | Definition of a VLAN X (x=VLAN No.) |
| ip address x.x.x.x y.y.y.y | Assigns the IP address "x.x.x.x" with the network mask "y.y.y.y" to the selected router interface. |
| ip dhcp pool pool_name | initiates the definition of a DHCP pool in global configuration mode |
| network x.x.x.x y.y.y.y | defines the IP range of the DHCP pool (network address and network mask) |
| default-router x.x.x.x | defines the default gateway for the DHCP pool (IP address) |
| dns-server x.x.x.x | defines the name server for the DHCP pool (IP address) |
| ip dhcp excluded-address x.x.x.x y.y. y.y | defines the range of IP addresses that should not be assigned via DHCP (start and end IP code). |
| access-list n permit x.x.x.x y.y.y.y | defines an Access list with the number "n" that allows packets from the described network area - "x.x.x.x" is the network address and "y.y.y.y" is the inverse network mask |
| Ip dns server | Configure router as dns server |
| ip host NAME IP | Configure mapping between hostname and ip |
| router rip | switches from global configuration mode to RIP configuration mode or sets up a RIP routing process. |
| version x | sets up the RIP version |
| network x.x.x.x | connects a network to the RIP routing process |
| sh ip rip database | outputs RIP information |
| sh ip route | outputs the routing table |
| show ip access-lists | displays the ip access lists with the individual matches (hits) |
| ip access-list extended acl_name | switches from global configuration mode to extended access list mode or sets up a new named extended access list. |
| permit¦deny protocol source [eq port] destination [eq port] [state] | defines a permit or deny rule within the access list, where protocol determines the protocol (udp/tcp/icmp), source describes the source range, and destination describes the target area. The areas are described via the network address and the inverse network mask. If the range to be specified is not defined more precisely, "any" can be used. Furthermore, ports and the status can be specified. The last rule should always be "deny ip any any". |
| ip access-group acl_name in¦out | attaches the extended named access list to an interface and determines the direction of action with in/out. |